| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-03 Broadband Speed, Upload, Latency, Jitter & Peak-Time Consistency US · United Kingdom · Germany · Snapshot checked October 2026 |
| --- | --- |

Purpose. This guide explains the performance numbers households should understand before judging a broadband plan or diagnosing a slow connection. It is designed for the ONOCRA Guides index and for Step 4/Step 5 evidence modules.

The ONOCRA rule: broadband quality is more than download speed

| Metric | Plain meaning | Why it matters |
| --- | --- | --- |
| Download speed | How quickly data reaches the home/device. | Streaming, downloads, app updates and many simultaneous downstream activities. |
| Upload speed | How quickly data leaves the home/device. | Video calls, cloud backup, creator uploads, remote work and sending large files. |
| Latency | Round-trip delay between the user and a remote service. | Interactive tasks such as calls, remote control and online gaming. |
| Jitter | How much latency varies from moment to moment. | Uneven delay can make live audio/video or gaming feel unstable even when average latency looks acceptable. |
| Packet loss | Data packets that do not arrive successfully. | Can harm real-time services and reduce effective throughput. |
| Peak-time consistency | How well performance holds up when the network is busy. | Separates a strong connection from one that only looks good in a quiet-time test. |

Why one impressive speed-test number can mislead

• A Wi-Fi test can be limited by the router, device, distance, walls or interference rather than the broadband line.

• Download may be excellent while upload is the real bottleneck.

• A test at 3am says little about evening congestion.

• A nearby test server can show low latency even when a particular game, work service or cloud platform is farther away or poorly routed.

• Background downloads, cloud sync, software updates or another household member can distort a result.

• Provider “up to”, estimated, normally available and minimum-guaranteed figures are not interchangeable terms.

Quick household test workflow

| A Identify the exact plan and its stated download/upload information. | B Test one capable device by Ethernet where practical. | C Repeat at several times, including the household’s busy period. |
| --- | --- | --- |
| D Record download, upload, latency and—where available—jitter/packet loss. | E Run a separate Wi-Fi test to measure the home wireless layer. | F Compare patterns over time, not only the single best result. |

| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-03 1 · Download & Upload Speed Measure both directions; do not infer upload from the download headline |
| --- | --- |

Download and upload are different resources

| Question | Download | Upload |
| --- | --- | --- |
| What is being measured? | Data received by the home/device. | Data sent from the home/device. |
| Common household pressure | Streaming, game/app downloads, web content, software updates. | Video meetings, cloud backup, photo/video uploads, sending files, live streaming. |
| Why plans can differ | Many consumer access networks historically devote more capacity to downstream traffic. | Upload capacity may be materially lower on asymmetric services; newer access generations can improve it. |
| What to record | Measured rate plus the plan’s stated/estimated figure. | Measured rate plus the plan’s stated/estimated figure—never assume symmetry. |
| What can distort a test? | Wi-Fi, device limits, local network use, server path, congestion. | The same factors plus any active backups/sync or upstream traffic. |

Symmetrical vs. asymmetrical service

A symmetrical plan offers the same nominal upstream and downstream rate. An asymmetrical plan provides different rates, usually a lower upload rate. Technology alone does not guarantee symmetry: the actual provider plan and provisioning determine what the customer receives.

The access line can be fast while the experience is still slow

• Wi-Fi can become the bottleneck on a fast fibre/cable plan, especially at distance or on older client hardware.

• A computer with an older Ethernet port, slow storage, high CPU load or a busy browser can cap a test.

• A VPN or security tunnel can add processing and routing overhead.

• The test server and wider internet route also affect the result; a speed test is not a direct measurement of every website or service.

• Other devices on the home network can consume capacity at the same time.

Household interpretation: ask what is competing at the same time

| Household pattern | Performance evidence to prioritise | Why |
| --- | --- | --- |
| Several 4K/HD streams and large downloads | Sustained download capacity + peak-time consistency | The issue is aggregate downstream demand, not only a one-device test. |
| Frequent video calls / remote work | Upload + latency + jitter + packet loss | Live two-way traffic is sensitive to both upstream capacity and delay stability. |
| Cloud backup / creators | Upload + long-duration consistency | A short download-focused test may hide slow upstream transfers. |
| Gaming | Latency + jitter + packet loss; then adequate throughput | Most games need modest throughput compared with downloads, but unstable delay can be noticeable. |
| Many smart devices | Overall stability + router/Wi-Fi quality | The broadband line and the home network must both remain healthy. |

| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-03 2 · Latency, Jitter, Packet Loss & Loaded Delay The responsiveness layer: why “fast Mbps” can still feel bad |
| --- | --- |

Four delay-quality concepts in plain words

| Metric | What it means | Household effect | How to read it |
| --- | --- | --- | --- |
| Latency | Round-trip time between the user and a destination. | Higher delay is more noticeable in calls, remote interaction and games. | Compare the same destination/test method at different times; lower and stable is generally preferable. |
| Jitter | Variation in latency from packet to packet or over short periods. | Can cause uneven audio/video timing and inconsistent gaming response. | A stable connection has less variation, not merely a good average. |
| Packet loss | Packets sent but not successfully received. | Can create glitches in real-time traffic and reduce effective throughput. | Persistent or rising loss deserves investigation; repeat tests and isolate Wi-Fi vs broadband. |
| Loaded latency | Latency while the connection is busy with heavy upload/download traffic. | Shows whether responsiveness collapses when someone else fills the line. | Useful for busy households; router queue management and access congestion can affect it. |

What can raise latency or jitter?

• Physical access technology and distance/propagation path.

• Congestion in the access network, backhaul or wider provider network.

• Wi-Fi interference, retransmissions or weak signal inside the home.

• Long or inefficient routing to the application/server.

• Heavy upload/download queues when a connection is saturated.

• Technology-specific effects: FWA radio/load, satellite orbit/network design, DSL error correction, and shared HFC/PON capacity.

Do not compare unlike tests

A 12 ms result to a nearby speed-test server cannot be compared directly with 45 ms to a distant game server. Use the same device, connection method, server or application target and similar test conditions when looking for trends.

Practical diagnostic pattern

| Pattern | Likely interpretation | Next check |
| --- | --- | --- |
| Ethernet good, Wi-Fi bad | Local wireless is the likely bottleneck. | Check placement, band/channel, backhaul and client capability. |
| Download good, upload poor | Upstream capacity or background upload may be limiting. | Pause backups/sync; compare the plan upload figure; retest. |
| Speed good, delay unstable at busy time | Congestion, queuing, radio variation or routing may be involved. | Compare quiet vs busy periods using the same wired test. |
| All metrics degrade together | Access/provider congestion or a line/signal fault is more plausible. | Check equipment status/provider notices and repeat wired tests. |

| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-03 3 · Peak-Time Consistency A connection should be judged when the household and network are actually busy |
| --- | --- |

Peak time is a performance dimension—not a separate speed tier

Broadband performance can change with local and provider-network load. For home services, Ofcom’s consumer guidance uses 8–10pm as the busy-time window for speed estimates from participating providers. The FCC Measuring Broadband America methodology has also measured fixed-broadband speed during defined evening peak hours. ONOCRA should therefore store time-of-day with every performance observation rather than treating a single test as timeless.

Technology context: where variation can enter

| Access family | What may stay fairly stable | What may vary with load / environment | Fair ONOCRA wording |
| --- | --- | --- | --- |
| DSL / FTTC | Line sync can be relatively stable on a healthy line. | Provider/backhaul congestion; line errors can also change actual throughput. | Separate sync/line estimate from real internet throughput. |
| Cable / HFC | Modem link may remain up. | Shared node capacity, upstream contention, RF impairment and provider congestion. | Compare busy-period results; do not infer from DOCSIS generation alone. |
| FTTP / PON | Optical link is generally stable once installed. | Shared PON segment and upstream provider capacity can still congest. | Fibre is not immune to oversubscription or routing limits. |
| 4G / 5G FWA | Gateway may keep a strong signal. | Cell/sector load, radio conditions, band changes, traffic management, backhaul. | Test at the exact premises during the intended usage hours. |
| Satellite | Terminal may retain a clear sky view. | Beam/area capacity, weather, network routing and constellation design. | Orbit type matters, but service load/capacity still needs real testing. |

Consistency is often more useful than the best result

• Record several comparable tests and look at the spread, not just the maximum.

• Keep the connection method consistent: Ethernet-to-router or Wi-Fi should not be mixed in the same trend without labels.

• Test the household’s real busy periods—after school, evening streaming, work calls or gaming sessions.

• If a provider supplies a personalised/guaranteed figure, preserve that evidence and compare like-for-like conditions.

• Do not turn crowdsourced regional test maps into a guarantee for one household; they are context, not address-specific performance evidence.

Regional consumer evidence

| Market | Useful official reference | What it contributes |
| --- | --- | --- |
| United States | FCC Measuring Broadband America | Definitions and measurement of download/upload, latency, packet loss and consistency across fixed broadband. |
| United Kingdom | Ofcom broadband speeds guidance / codes | Busy-time speed estimates, minimum guaranteed speed information for participating providers, and Ethernet-first testing advice. |
| Germany | Bundesnetzagentur Breitbandmessung | Provider- and technology-neutral measurement; desktop evidence process comparing actual with contractually agreed rates. |

| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-03 4 · How to Test Broadband Fairly at Home Separate the internet access line from Wi-Fi before drawing conclusions |
| --- | --- |

Recommended ONOCRA test sequence

| 1 | Record the plan | Write down the exact plan name and its stated download/upload information. Save any minimum/normal/estimated speed statement supplied at sale. |
| --- | --- | --- |
| 2 | Use one capable device | Prefer a recent computer with a known Ethernet link speed. Connect directly to the router/gateway where practical. |
| 3 | Pause avoidable traffic | Stop large downloads, backups and cloud sync on other household devices if the goal is to measure line capacity. |
| 4 | Use the same test method | Use the same speed-test service/server when comparing time periods. Record whether a VPN is active. |
| 5 | Repeat across time | Run multiple tests, including the household’s normal busy period and a quieter period. Record date/time. |
| 6 | Test Wi-Fi separately | After the wired baseline, test the places where people actually use Wi-Fi. This reveals the in-home layer. |
| 7 | Keep evidence | Save screenshots or the official measurement report if you may need to discuss underperformance with the provider. |

Common testing mistakes

• Running one Wi-Fi test beside the router and assuming it proves whole-home performance.

• Comparing tests from different devices, servers or VPN states without noting the change.

• Testing only when the network is quiet.

• Leaving a cloud backup or game download running during a “line speed” test.

• Ignoring upload, latency and consistency because the download number looks high.

• Rebooting or factory-resetting equipment before recording fault lights, sync/signal status and baseline results.

Testing should create evidence, not chase a single perfect number. If repeated wired measurements still show a persistent problem, move to the evidence checklist on the next page before contacting the provider.

| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-03 5 · Evidence to Capture Before Escalating Build a reproducible record before asking the provider to investigate |
| --- | --- |

Before contacting the provider

| Evidence to capture | Why it helps |
| --- | --- |
| Exact plan / contract speed statement | Defines the service you are comparing against. |
| Date/time + test method + Ethernet/Wi-Fi label | Makes repeated measurements comparable. |
| Download + upload + latency; jitter/loss if available | Shows whether the issue is capacity, responsiveness or both. |
| Router/modem/ONT/gateway status | Can reveal line, optical, coax or radio problems. |
| Several tests across days/times | Distinguishes a persistent issue from a one-off event. |
| Official measurement protocol/report where applicable | May be required for formal rights or remedies in some markets. |

How to describe the problem clearly

• State whether the issue affects Ethernet, Wi-Fi or both.

• Give the exact plan/tier and the provider-supplied speed information.

• Provide several dated test results rather than only the worst or best result.

• Say whether the problem is download, upload, latency/jitter, packet loss or time-of-day consistency.

• Note any modem/ONT/gateway warning lights, sync/signal changes or provider outage messages.

• For FWA or satellite, include gateway/terminal placement and whether signal/obstruction conditions changed.

Do not destroy useful evidence first

Avoid a factory reset before recording equipment status, configuration and test results. A normal restart may be reasonable when the provider or equipment maker recommends it, but preserve the fault evidence first—especially if the problem is intermittent.

When a formal measurement procedure matters

For ordinary troubleshooting, several well-controlled tests can show a useful pattern. If the consumer intends to rely on contractual rights or claim underperformance, use the current official measurement procedure for that country. Germany, for example, provides a formal fixed-broadband desktop measurement process; UK rights depend on the provider/code and contract information; US remedies depend on provider terms and applicable consumer rules.

| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-03 6 · ONOCRA Step 4 Evidence Map Structured fields for fair, reproducible performance guidance |
| --- | --- |

Step 4 extraction fields

| Field | What ONOCRA should store | Editorial rule |
| --- | --- | --- |
| plan_identity | Provider · plan/tier · access technology · region/address scope | Never compare a measurement to an unidentified plan. |
| advertised_or_estimated_download | Exact published/quoted figure + label used by source | Preserve “advertised”, “estimated”, “normally available”, “minimum guaranteed” wording. |
| advertised_or_estimated_upload | Exact published/quoted figure + label | Do not infer upload from download. |
| measured_download_upload | Values + date/time + test method + Ethernet/Wi-Fi + device | A measurement without context is weak evidence. |
| latency_jitter_packet_loss | Values where the method reports them | Keep destination/server/method comparable across observations. |
| peak_time_observation | Busy-period values or range | Store the time window; do not merge quiet- and peak-time tests. |
| wifi_separated | Yes/no + Wi-Fi generation/band/location if relevant | Prevents blaming broadband for a local wireless bottleneck. |
| evidence_source_confidence | Official protocol / provider / independent test / household observation | Different evidence types should not be presented as equivalent. |
| observation_date | Date checked | Performance claims age; preserve snapshot date. |

Interpretation safeguards

• Keep provider/plan identity attached to every performance observation.

• Do not compare Wi-Fi-only measurements directly with wired broadband measurements unless the distinction is visible.

• Preserve test date/time because network conditions and plan provisioning can change.

• Use official measurement procedures when a consumer intends to claim contractual underperformance.

• Treat crowdsourced or third-party measurements as context, not as a guarantee for one household.

| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-03 7 · Verification Sources & Terms Official consumer-performance references for US · UK · Germany |
| --- | --- |

Official verification sources

| Source | Use in this guide | Link |
| --- | --- | --- |
| United States — FCC Measuring Broadband America | Defines and measures download/upload, latency and packet loss; explains consistency and peak-hour methodology. | Open official source |
| United Kingdom — Ofcom: Broadband speeds — what you need to know | Consumer guidance on busy-time estimates, minimum guaranteed speeds and Ethernet-first testing. | Open official source |
| United Kingdom — Ofcom broadband & Wi-Fi help | Advises repeated tests, notes day/time variation and separates broadband from Wi-Fi limitations. | Open official source |
| Germany — Bundesnetzagentur Breitbandmessung | Official provider-/technology-neutral speed measurement; compares actual with contractually agreed rates. | Open official source |
| Germany — Bundesnetzagentur Internetgeschwindigkeit | Official fixed/mobile evidence procedures and consumer rights when performance materially deviates from contract. | Open official source |

Terms in plain words

| Term | Meaning |
| --- | --- |
| Mbps / Gbps | Megabits or gigabits per second: rates used to describe data transfer speed. |
| Throughput | The actual rate achieved during a transfer or measurement under stated conditions. |
| Latency | Delay, usually expressed as a round-trip time in milliseconds. |
| Jitter | Variation in delay over time. |
| Packet loss | The share of transmitted packets that do not successfully reach the destination. |
| Peak time | A defined busy-use period; the exact regulatory/testing window can differ by market or methodology. |
| Ethernet baseline | A wired test used to reduce Wi-Fi as a confounding factor. |
| Wi-Fi bottleneck | When the local wireless link is slower or less stable than the broadband access line. |

Editorial safeguard. ONOCRA does not run a laboratory and should not present one household speed test as a universal provider score. Performance observations must be attributed, dated and labelled by test method. Regulatory rights and measurement protocols differ by country and can change; verify the current official process before advising a contractual remedy.