DSL / Copper Broadband vs. Satellite Internet

An ONOCRA comparison framework for household broadband choices

This guide compares two very different ways to deliver home internet: DSL broadband over telephone copper pairs, including ADSL/VDSL and fibre-to-the-cabinet (FTTC) where copper remains on the final access segment, and satellite internet, where a user terminal communicates through an orbital network. It helps you decide which questions matter before you choose a provider or plan.

How to read it. Neither technology is “better” in every case. DSL performance is strongly shaped by copper line length, condition and network design; satellite performance is strongly shaped by orbit type, sky visibility, capacity and service design. This is qualitative technical context only. These are not measured provider or product results, and we do not publish invented prices or scores. Speeds, prices and availability differ by provider, region and plan.

## Evidence key

| Provider claim | What the provider or manufacturer says. Useful, but verify it. |
| --- | --- |
| Independent testing | Measurements by third parties. Check the date, location and method. |
| Editorial interpretation | ONOCRA’s reading of the evidence. Marked as such, never presented as a measurement. |
| Unverified | No reliable evidence found yet. We say so rather than guess. |

# Full comparison

| Factor | DSL / copper broadband | Satellite internet | What to verify |
| --- | --- | --- | --- |
| 1 · Technology & network design |  |  |  |
| How it works | Broadband runs over telephone copper pairs. ADSL may run from an exchange; VDSL commonly uses fibre to a street cabinet with copper for the final segment (FTTC). A DSL modem or gateway terminates the line at the premises. | A user terminal (dish or flat-panel antenna) communicates with satellites and the operator network. The exact path depends on the constellation, orbit and ground infrastructure. | Ask whether the wired service is ADSL, VDSL or FTTC and how much copper remains. For satellite, ask which orbit class serves the address and what equipment is supplied. |
| Network generation / orbit type | ADSL, ADSL2+ and VDSL are different DSL generations with different capabilities. FTTC is not full fibre to the premises because copper remains between the cabinet and the home. | LEO, MEO and GEO are different orbital architectures. Household services are commonly LEO or GEO; they should not be treated as equivalent because signal path, delay and network design differ. | For DSL, ask which variant serves your line and whether migration to full fibre is planned. For satellite, verify the orbit class and the provider’s typical latency guidance. |
| Shared vs. dedicated capacity | Each premises normally has its own copper pair on the access line, but the broadband network upstream is shared. Capacity and congestion depend on the provider network, cabinet/exchange backhaul and local design. | Capacity is shared across the serving beam/cell or constellation resources. Busy areas may be capacity-limited or subject to sign-up controls or priority policies. | For DSL, request an address-specific line estimate and typical busy-period information. For satellite, ask about local/beam capacity, priority rules and any capacity-based sign-up restrictions. |
| Signal path & environment | Copper DSL signals weaken with line length and can be affected by electrical interference, poor joints, corrosion, water ingress and in-home wiring. The line does not require a radio path or sky view. | The terminal needs the required view of the sky. Trees, buildings, roof position and other obstructions can interrupt or degrade the link, depending on the system. | For DSL, ask who maintains the copper path and internal wiring and whether line diagnostics are available. For satellite, run the provider’s obstruction check or site survey before ordering. |
| Infrastructure dependence | Depends on the copper access network, exchange or street cabinet, powered equipment where used, backhaul/core network and the physical condition of the line to the premises. | Depends on the satellite constellation, operator network and ground infrastructure/backhaul; some systems also use inter-satellite links. | Ask how each service handles outages, maintenance, backhaul/core failures and customer-equipment faults. |
| 2 · Speed & performance |  |  |  |
| Download capacity | Capacity varies widely by DSL variant, line length and copper condition. VDSL on a short, healthy line may be materially faster than ADSL, but address-specific estimates matter more than headline technology names. | Can offer useful to high download speeds depending on the system, plan, location and capacity. Results vary. | Check provider-published advertised, typical and minimum information where available. For DSL, use an address-specific estimate; for satellite, compare results for the relevant service area and plan. |
| Upload capacity | Upload is usually much lower than download, especially on ADSL. VDSL may improve upstream capacity but remains dependent on the specific line and service profile. | Upload is often lower than download and varies by system, plan, terminal conditions and capacity. | Look for the stated upload figure. Do not infer upload from download, and test during any trial or return period if possible. |
| Latency & jitter | Can be suitable for calls and gaming when the line is stable and the provider network is well engineered. Latency and jitter may rise with line errors, retransmissions, congestion or poor routing. | Latency depends strongly on orbit and network design. GEO has much higher propagation delay than LEO; LEO can be closer to terrestrial broadband but still varies with routing, handovers and congestion. | Compare independent latency and jitter measurements from several times of day. For satellite, do not compare LEO and GEO as if they are equivalent. |
| Peak-time consistency | The local DSL sync rate may remain fairly steady, but congestion can still occur upstream in the provider network. A degraded copper line may also retrain or reduce its rate. | Depends on how many users share available capacity in the serving area/beam and on operator policy. Busy periods can reduce throughput or priority. | Ask whether the provider publishes typical busy-period performance and test during your own normal peak hours. |
| Weather sensitivity | DSL is not affected by radio rain fade, but outdoor copper plant, joints and cabinets can still be affected by water ingress, storms, lightning, physical damage or power loss. | Rain, snow and severe weather can affect some satellite links, with impact varying by frequency, terminal, orbit and operator design. | For DSL, ask about line/cabinet fault restoration. For satellite, check terminal weather features, mounting requirements and any provider guidance for rain or snow. |
| Stability & interruptions | Noisy or deteriorated copper can cause retrains, dropouts or reduced sync rates. Modem/router faults and provider-network outages can also interrupt service. | May experience interruptions from obstructions, weather, capacity conditions, terminal issues, satellite handovers or operator-network faults, depending on the architecture. | Ask about outage handling, service-restoration targets, line/terminal diagnostics and support arrangements. |
| 3 · Everyday household use |  |  |  |
| Video calls & remote work | Can work well on a healthy VDSL line, but several simultaneous calls, screen sharing or large uploads may expose limited upstream capacity or instability. | Interactive performance depends strongly on latency and stability. GEO delay can be noticeable for calls; LEO can be more suitable but results vary by service and conditions. | Test calls at busy times and compare real upload, latency and jitter rather than relying on download speed alone. |
| Cloud backup & photo uploads | Lower DSL upload rates can make large first backups or frequent photo/video uploads slow, especially on ADSL. | Upload speed, priority/fair-use rules and capacity decide how practical large backups are. | Estimate your regular upload volume and compare it with upload performance, data/priority rules and busy-period consistency. |
| 4K streaming & downloads | Depends on the DSL variant, real line rate and number of simultaneous users. Slower lines may struggle with several high-bitrate streams or large downloads at once. | Can work where speeds, stability and data/priority allowances are adequate. Capacity rules or congestion can matter during heavy use. | Plan for simultaneous household use, not one device, and check busy-period performance plus any data or priority limits. |
| Online gaming | Can be suitable when latency and line stability are good. Noisy copper, retransmissions or heavy household use can increase delay and jitter. | Orbit type is critical. GEO latency is a poor fit for latency-sensitive gaming; LEO may be more suitable, subject to local routing, capacity and stability. | Test latency and jitter during usual gaming hours. Use Ethernet to the home router where possible so Wi-Fi does not distort the comparison. |
| Many devices / smart home | A slower DSL line can become the bottleneck when many devices stream, update or back up at once. Router and Wi-Fi capability remain separate from the broadband line. | The supplied router/Wi-Fi system and satellite capacity both matter. Some operators allow third-party routers or bypass/bridge modes; others are more restricted. | Count active devices and check router/gateway capabilities, Ethernet ports, mesh support and whether your own router can be used. |
| Hosting, port forwarding & static IP | Public IPv4, static IP, IPv6 and port-forwarding options depend on the provider and plan. Some DSL services may still use CGNAT or restrict inbound access. | Some satellite services use CGNAT/shared addressing. Public IPv4, IPv6, port forwarding and static-IP options vary by operator and plan. | If you need remote access, cameras, servers or inbound VPNs, verify CGNAT/public-IP/IPv6/static-IP and port-forwarding policy before ordering. |
| 4 · Availability & installation |  |  |  |
| Availability | Available where an active copper telephone access network and DSL service remain. Availability and supported rates depend on the exact line and local migration/retirement plans. | Can have wide geographic reach, but service area, regulation, plan eligibility, capacity and obstruction conditions still apply. | Check exact-address eligibility. For DSL, verify the address-specific line estimate and any copper-retirement plan. For satellite, verify service-area eligibility and obstruction. |
| Installation | Often uses an existing telephone pair, so activation may be simple where a usable line is present. New line work, repairs or provider migration can add delay. | May be self-install or professionally installed. Outdoor mounting must provide the required sky view and may need drilling, roof/pole/wall access or specialist work. | Ask what work is required, whether an engineer or external mount is needed, who installs it, and what happens if the site is unsuitable. |
| Renters & multi-dwelling buildings | Existing telephone wiring may simplify installation, but landlord/building permission may still be needed for new cabling, repairs or shared-building work. | An outdoor satellite terminal or mount may require landlord, building-owner or association approval, especially on shared roofs/facades. | Ask before drilling, changing shared building infrastructure or fitting external equipment. |
| Equipment in your home | DSL modem or modem-router; some installations may also use microfilters/splitters depending on the telephone/voice arrangement. Your own router may be supported in some configurations. | A user terminal, power supply, cabling/mounting hardware and router/gateway. Exact components vary by operator and product generation. | Ask for the full equipment list, ownership/rental status, replacement cost, compatibility and return requirements. |
| Power & backup | The DSL modem/router needs household power. Street cabinets used for FTTC and provider-network equipment also need power, while the copper pair itself is passive. | The satellite terminal and router need power; terminal consumption can be materially higher than that of a typical router and may vary with weather/heating features. | Check device power consumption and whether backup power is practical for the household use case. Remember upstream network infrastructure also needs power. |
| Time to get connected | Can be quick where an active copper line is already present; faults, new line work or migrations can delay service. | Depends on service eligibility, terminal delivery, capacity and whether professional installation is needed. | Request realistic delivery, installation and activation dates before relying on the service for work or study. |
| 5 · Cost, contract & consumer protection |  |  |  |
| Equipment & installation cost | May include activation, engineer, line-repair or modem/router charges depending on provider and line condition. | The terminal may be rented, purchased or included. Mounting, shipping, installation and return costs can materially affect the initial outlay. | Add up activation, equipment, mounting, delivery, installation, replacement and return charges before comparing the offers. |
| Monthly price & total cost | Pricing varies by provider and market. Legacy DSL is not automatically cheaper; compare regular price, bundled-line charges and any planned migration terms. | Pricing varies by operator, region and plan. Compare ongoing service cost together with terminal/equipment charges and any priority-data options. | Compare first-year and ongoing total cost, including post-promotion pricing, equipment charges and any annual increases. |
| Data limits & fair-use rules | DSL plans may be unlimited or may include fair-use/traffic-management rules depending on provider and market. Check the exact plan rather than assuming. | Data allowances, priority data, congestion management, throttling or fair-use rules may apply depending on plan and operator. | Ask for the exact policy: allowance, priority threshold, what changes when reached, and whether there are extra charges or congestion-based restrictions. |
| Trial & cancellation terms | Contract lengths, migration/closure terms, cooling-off rights and early-termination conditions vary. A copper-retirement or technology migration may change the service path. | Trial, return and cancellation terms vary. Terminal-return, mounting and restocking conditions can be important. | Read trial/return/cancellation steps before ordering and note deadlines, equipment condition requirements and technology-migration terms. |
| Consumer information | Local rules may require providers to disclose prices, speeds, contract terms and data policies. Exact format and consumer protections differ by country. | Pre-contract disclosures, service descriptions, cancellation rights and equipment terms vary by country and operator. | Keep a copy of the provider’s current pre-contract information, service description and equipment/return terms. |
| 6 · Longevity & value |  |  |  |
| Upgrade path | DSL improvement is constrained by the copper access line. Major gains typically require fibre closer to, or all the way to, the premises; many markets are progressively migrating away from copper. | Can improve through newer satellites, constellation capacity, ground-network upgrades and terminal generations, but customer hardware may need replacement. | Ask what concrete technology upgrades or migrations are planned for your address/service area and whether new customer equipment would be required. |
| Future-proofing (editorial view) | DSL/copper is an older access family in many markets and is being migrated or retired in some areas. Suitability depends on the current line, price and realistic local upgrade path. | Can be a practical long-term option where terrestrial services are limited, but suitability depends on local capacity, operator investment, terminal evolution and service terms. | Treat future-proofing as editorial interpretation, not a guarantee. Re-check copper-retirement plans, local capacity and provider roadmaps before a long commitment. |
| Energy & environment (editorial view) | Energy use depends on the DSL access network, street cabinets/exchanges where applicable, premises equipment and how efficiently the provider operates the network. | Depends on terminal power, satellite/ground infrastructure, launches, equipment lifecycle and the operator’s architecture. Simple universal comparisons are not appropriate. | Use provider/operator lifecycle or sustainability disclosures where available and avoid unsupported environmental superiority claims. |

# Quick fit guide (editorial interpretation)

These are general tendencies, not guarantees. Your address, provider and plan decide the real result.

| DSL / copper broadband tends to suit households that… | Satellite internet tends to suit households that… |
| --- | --- |
| Have light to moderate use and an address-specific DSL estimate that meets their needs. Live where copper broadband remains available and stable, but newer fixed networks are not yet practical. Prefer an existing wired line with no outdoor radio/satellite equipment. Find the DSL plan clearly priced and sufficiently reliable for the expected remaining service life. | Live where terrestrial fixed broadband is unavailable, difficult or uneconomic. Have the required clear sky view and can accommodate a dish or flat-panel terminal. Can accept the latency characteristics, equipment/power requirements and data/priority policy of the chosen orbit/service. Value geographic reach more than the convenience of an existing wired access line. |

# Questions to ask before you sign

- Is the wired service ADSL, VDSL or FTTC, and what address-specific line estimate and copper path apply?

- For satellite, is the service LEO, MEO or GEO, and what latency range should I expect?

- What are the real download and upload expectations for each option at my address, including busy-period performance?

- For DSL, what is the line condition and local copper-retirement/upgrade path? For satellite, has a sky-obstruction check been completed?

- What is the total cost now and after any promotion, including activation, modem/router or terminal, mounting, delivery and return charges?

- Are there data caps, priority-data thresholds, throttling, traffic-management or fair-use terms?

- Can I use my own router, and what are the CGNAT/public-IP/IPv6/port-forwarding options if I need cameras, NAS or remote access?

- What are the installation, landlord/building-permission, fault-restoration, power-backup and cancellation/return arrangements?

## Short glossary

- DSL: digital subscriber line. Broadband carried over telephone copper pairs.

- ADSL / VDSL: DSL variants. VDSL generally supports higher rates over shorter copper runs than ADSL.

- FTTC: fibre to the cabinet. Fibre reaches a street cabinet; copper remains from the cabinet to the premises.

- LEO / MEO / GEO: low, medium and geostationary Earth orbit families. Orbit height and architecture strongly influence propagation delay and system design.

- Satellite terminal: the dish or flat-panel antenna, power supply and associated equipment used to connect the home to the satellite network.

- Obstruction: a tree, building, roof feature or other object that blocks the terminal’s required view of the sky.

- Latency / jitter: delay in the connection, and how much that delay varies over time.

This document is general information, not a lab test or a provider recommendation. Verify technology, line or sky conditions, speeds, latency, prices, data policies, equipment, installation requirements and exact-address availability with the provider before ordering.