| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-05 Public IP, CGNAT, IPv6, Port Forwarding & Remote Access Home Internet & Wi-Fi · Decision & evidence guide |
| --- | --- |

Purpose. This guide explains why a fast broadband connection can still behave very differently for remote cameras, NAS access, self-hosted services, online gaming and inbound VPN connections. It separates public addressing from private addressing, explains carrier-grade NAT (CGNAT), and shows safer alternatives to simply opening ports.

ONOCRA rule: Never promise that “port forwarding will work” until the provider’s IPv4/IPv6 addressing, router mode and inbound-access policy are verified for the exact plan.

## The seven terms households should understand

| Term | Plain meaning | Why it matters |
| --- | --- | --- |
| Private IPv4 | Address used inside the home, commonly from RFC 1918 ranges. | Not directly reachable from the public internet by itself. |
| Public IPv4 | Globally routable IPv4 address assigned at or upstream of the customer connection. | Traditional inbound IPv4 hosting generally needs a usable public address plus firewall/NAT configuration. |
| CGNAT | The ISP shares one or more public IPv4 addresses among many customers using carrier-grade translation. | The customer usually cannot create a normal inbound IPv4 mapping through the ISP’s translator. |
| IPv6 | Newer Internet Protocol with a vastly larger address space. | Devices can have globally routable IPv6 addresses, but the router firewall still controls unsolicited inbound traffic. |
| Port forwarding | Router rule sending traffic arriving on a chosen public-side port to a device inside the home. | Useful for some self-hosted services; it also creates exposure that must be secured. |
| Double NAT | Two separate NAT routers are in series, for example an ISP gateway plus a second router. | Can complicate inbound access and gaming even without CGNAT. |
| Remote-access relay/tunnel | A provider or trusted service brokers an outbound encrypted connection from the home device. | Often works without inbound port forwarding and can be safer/easier for households. |

## The key distinction

NAT is not the same thing as a firewall, and IPv6 does not mean “no security.” A household may have public IPv4, CGNAT, IPv6, both IPv4 and IPv6, or provider-specific combinations. ONOCRA should record the actual plan behaviour rather than infer it from the access technology.

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 1

| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-05 1 · How to identify what your connection is actually using Home Internet & Wi-Fi · Decision & evidence guide |
| --- | --- |

## A safe household diagnostic sequence

| Step | Check | Interpretation / caution |
| --- | --- | --- |
| 1 | Open the router/gateway internet-status page and record the WAN IPv4 address and whether IPv6 is enabled. | Do not post the full public address in screenshots. Record only what is needed for diagnosis. |
| 2 | If the WAN IPv4 is in 10.0.0.0/8, 172.16.0.0/12 or 192.168.0.0/16, there is private addressing upstream or another NAT layer. | This may be double NAT or ISP-side translation; it is not by itself proof of RFC 6598 CGNAT. |
| 3 | If the WAN IPv4 falls inside 100.64.0.0/10, treat CGNAT/shared address space as the leading explanation. | RFC 6598 reserves this block specifically as Shared Address Space for service-provider CGN use. |
| 4 | Compare the router WAN IPv4 with the public IPv4 seen by a reputable “what is my IP” service. | If they differ, an upstream NAT/translation layer is likely. Some gateways hide details, so provider confirmation remains authoritative. |
| 5 | Check whether the router receives an IPv6 prefix and whether client devices have global IPv6 connectivity. | A local/link-local or unique-local IPv6 address is not proof of global IPv6 reachability. |
| 6 | If you use an ISP gateway plus your own router, check bridge/passthrough/access-point mode. | Two routing layers can create double NAT. Do not change modes until ISP voice/IPTV/authentication requirements are understood. |
| 7 | Ask the provider directly whether the plan offers public dynamic IPv4, static IPv4, CGNAT, native IPv6, and any inbound-port restrictions. | Provider/plan evidence overrides assumptions based on country, brand or access technology. |

## Recognise the special address ranges

| Range | Meaning | ONOCRA interpretation |
| --- | --- | --- |
| 10.0.0.0/8 · 172.16.0.0/12 · 192.168.0.0/16 | RFC 1918 private IPv4 | Normal inside homes; if shown as the ISP-facing WAN address, another NAT layer exists upstream. |
| 100.64.0.0/10 | RFC 6598 Shared Address Space | Strong CGNAT indicator when used on the provider-facing side. |
| 127.0.0.0/8 | Loopback | Local host only; not an ISP-assigned WAN address. |
| 169.254.0.0/16 | IPv4 link-local | Often indicates failure to obtain normal IPv4 configuration; do not confuse with CGNAT. |
| fe80::/10 | IPv6 link-local | Works only on the local link; not globally routable. |
| fc00::/7 | IPv6 Unique Local Address space | Intended for local communications, not normal global internet routing. |

Privacy safeguard: for support screenshots, redact public IP addresses, device serial numbers, Wi-Fi passwords, router admin credentials and QR codes.

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 2

| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-05 2 · What CGNAT, public IPv4 and IPv6 change in everyday use Home Internet & Wi-Fi · Decision & evidence guide |
| --- | --- |

## Most internet use works normally behind CGNAT

| Use case | Usually works behind CGNAT? | What can change |
| --- | --- | --- |
| Web browsing / streaming / messaging | Yes | These are normally outbound client connections. |
| Video calls | Usually | Quality depends more on upload, latency, jitter and NAT traversal. Some peer-to-peer paths may fall back to relays. |
| Cloud-managed cameras / smart-home apps | Usually | Many products use vendor cloud relays, so no manual inbound port is required. Verify the product’s security/update model. |
| Online gaming | Often | Matchmaking can work, but NAT type, peer hosting and specific game/platform behaviour may differ. |
| Remote access to a NAS / home server over IPv4 | Not by ordinary port forwarding if the ISP uses CGNAT | Use a provider public-IP option, IPv6 with secure firewall rules, or a trusted VPN/tunnel/relay design. |
| Hosting a VPN server at home | May fail over inbound IPv4 under CGNAT | A tunnel initiated outbound from home or native IPv6 can be alternatives. |
| Port-forwarded CCTV/NVR | Not by normal IPv4 forwarding under CGNAT | Prefer vendor-supported secure remote access or a VPN; directly exposing camera admin interfaces is high risk. |
| Business / advanced remote access | Plan-dependent | Static/public addressing, reverse DNS, firewall policy and terms of service can matter. |

## Access technology does not guarantee address type

| Technology | Common tendency worth checking | Do not assume |
| --- | --- | --- |
| DSL / Copper | Public IPv4 may exist on many legacy plans; IPv6 and static-IP options vary. | That every DSL customer gets a public IPv4. |
| Cable / HFC | Provider gateway mode, IPv6 and IPv4 policy are plan/provider specific. | That bridge mode automatically removes upstream CGNAT. |
| Full Fibre / FTTP | Modern networks often support IPv6; public IPv4 policy still varies. | That fibre automatically means static/public IPv4. |
| 4G / 5G FWA | CGNAT is common enough to be an explicit pre-purchase check. | That mobile/FWA always uses CGNAT or can never provide public addressing. |
| Satellite | Addressing and inbound policies can vary by operator, plan and region. | That every satellite service behaves the same for public IPv4 or IPv6. |

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 3

| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-05 3 · Port forwarding, IPv6 and safer remote access Home Internet & Wi-Fi · Decision & evidence guide |
| --- | --- |

## Port forwarding is a capability, not the default recommendation

| Option | When it can fit | Main safeguards / limitations |
| --- | --- | --- |
| Public dynamic IPv4 + port forward | A specific well-maintained service needs inbound IPv4 and the public address may change. | Use a strong authenticated service, minimum ports, patched software and dynamic DNS if needed. Do not expose router admin pages. |
| Static public IPv4 | Business/advanced use where a stable address is justified. | May cost extra; a static address does not make the service secure. Firewall and service hardening remain essential. |
| Native IPv6 inbound | Both provider and destination support global IPv6. | No NAT mapping is required, but the router firewall must deliberately allow only the required traffic. Verify prefix stability and client support. |
| VPN server on the home router/NAS | Trusted remote access into the home network. | Prefer modern authenticated VPN protocols; expose only the VPN endpoint and keep firmware/current software updated. |
| Outbound mesh VPN / tunnel | CGNAT blocks inbound IPv4 or the user wants simpler remote access. | The home device initiates outbound connectivity; trust, account security and service dependency must be considered. |
| Vendor cloud relay | Cameras/NAS/smart-home products designed for app-based remote access. | Convenient but dependent on vendor security, account MFA, support lifetime and cloud availability. |
| DMZ host / “expose all ports” | Rare troubleshooting only, if ever. | High exposure. ONOCRA should not recommend this as a routine fix. |
| UPnP automatic mappings | Consumer apps/games may request ports automatically. | Convenient but reduces visibility/control. Keep router firmware current and disable if not needed or if the threat model calls for stricter control. |

## IPv6: the important security correction

IPv6 removes the IPv4 address-shortage reason for NAT, but it does not remove the need for a stateful firewall. A global IPv6 address can be routable in principle; unsolicited inbound traffic should still be blocked unless a deliberate rule allows it. Do not copy an IPv4 “open a port” tutorial blindly into IPv6.

## If remote access suddenly stops

• Check whether the provider changed the WAN IPv4 address, moved the plan behind CGNAT, changed IPv6 prefix delegation, or replaced the gateway.

• Check double NAT after adding mesh or a second router.

• Check that the target device still has the expected local address; use DHCP reservations where appropriate.

• Check firewall rules and service listening ports before changing security settings.

• Prefer provider or vendor documentation over generic “open these ports” forum instructions.

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 4

| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-05 4 · Regional and provider verification — US / UK / Germany Home Internet & Wi-Fi · Decision & evidence guide |
| --- | --- |

There is no ONOCRA country-wide assumption for public IPv4, CGNAT, static IP or IPv6. These are provider-, network- and sometimes plan-specific features. The evidence should therefore be captured at the offer/plan level and dated.

| Market | What ONOCRA should verify | Evidence hierarchy |
| --- | --- | --- |
| United States | Whether the exact residential plan uses public IPv4 or CGNAT; whether native IPv6 is provided; static-IP availability/eligibility; gateway bridge/passthrough behaviour; any inbound-service restrictions. | 1) Provider plan/support page 2) Gateway status/plan documentation 3) Provider support confirmation 4) User measurement as supporting evidence. |
| United Kingdom | Public/shared IPv4 behaviour, IPv6 status, router/modem mode, static-IP option and whether features differ between residential/business tiers. | Provider documentation and current support confirmation. Do not infer from Ofcom availability data; address availability and IP-address policy are separate questions. |
| Germany | IPv4/IPv6 mode, DS-Lite/CGNAT or public-address options where applicable, router freedom/own-router implications, static-IP availability and inbound restrictions. | Provider documentation for the exact tariff plus router WAN status. Record whether a public IPv4 add-on or business tier is required. |

## Questions to ask before buying when remote access matters

| # | Question |
| --- | --- |
| 1 | Does this exact plan give me a publicly reachable IPv4 address, or is IPv4 behind CGNAT/shared addressing? |
| 2 | Is the public IPv4 dynamic or static, and is there an extra charge or eligibility condition? |
| 3 | Does the plan provide native IPv6 and prefix delegation to my own router? |
| 4 | Can I use bridge/passthrough mode, or must the ISP gateway remain the routing device? |
| 5 | Are inbound ports filtered or blocked by the provider? |
| 6 | Can I run a home VPN server, NAS or other self-hosted service under the terms of service? |
| 7 | If the provider uses CGNAT, is there a supported public-IP option? |
| 8 | Will changing address, plan, gateway or access technology change the IP policy? |
| 9 | For FWA/satellite, does the policy differ by region or residential/business plan? |
| 10 | What support evidence can I save now: plan page, support article, chat transcript or written confirmation? |

ONOCRA buying rule: If inbound access is essential, public-address capability is a suitability gate — not a minor specification to discover after installation.

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 5

| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-05 5 · Step 4 evidence fields, glossary & standards sources Home Internet & Wi-Fi · Decision & evidence guide |
| --- | --- |

## Step 4 structured evidence handoff

| Field group | Standard fields | Why Step 4 needs it |
| --- | --- | --- |
| Addressing & inbound | ipv4_mode · ipv4_public_status · cgnat_status · ipv6_status · inbound_ipv4_supported · inbound_ipv6_supported · provider_port_filtering | Distinguishes public IPv4, CGNAT and IPv6 and whether self-hosting/remote access is feasible. |
| Topology & stability | isp_gateway_required · own_router_supported · bridge_passthrough · double_nat_risk · ipv4_dynamic_static · ipv6_prefix_stability · static_ip_option | Separates ISP-side CGNAT from in-home double NAT and records whether addresses/prefixes may change. |
| Remote-access fit | nas_remote_access · vpn_server_fit · gaming_nat_note · camera_remote_access_note | Lets Step 4 show use-case consequences rather than protocol jargon. |
| Security | firewall_default · upnp_note · vendor_relay_note · mfa_available · update_support_note | Prevents “make it reachable” advice from bypassing security. |
| Evidence | provider_source_url · observation_date · plan_name · region · confidence | IP policy can change by provider, plan and date; evidence must be attributable. |

## Terms in plain words

| Term | Meaning |
| --- | --- |
| RFC 1918 | Private IPv4 address ranges commonly used inside homes and organisations. |
| RFC 6598 Shared Address Space | 100.64.0.0/10, reserved for service-provider shared addressing such as CGN deployments. |
| NAT | Address/port translation between networks. Consumer routers commonly use it for IPv4. |
| CGNAT | Carrier-grade NAT performed in the provider network, outside the customer router. |
| IPv6 | Internet Protocol version 6; designed as IPv4’s successor and using a much larger address space. |
| Port forwarding | Rule that directs selected inbound traffic to a chosen internal device/service. |
| Double NAT | Two NAT routing layers in series, often ISP gateway plus a second router. |
| Prefix delegation | Method by which an ISP delegates an IPv6 prefix to the customer router for downstream networks. |

## Standards & verification sources — checked 20 October 2026

IETF / RFC Editor — RFC 6598: IANA-Reserved IPv4 Prefix for Shared Address Space: Open official source

IETF / RFC Editor — RFC 1918: Address Allocation for Private Internets: Open official source

IETF / RFC Editor — RFC 8200: Internet Protocol, Version 6 (IPv6) Specification: Open official source

IETF / RFC Editor — RFC 4193: Unique Local IPv6 Unicast Addresses: Open official source

General information only. ONOCRA does not operate a test laboratory and should not publish a provider’s IP-addressing policy without dated evidence. Security-sensitive configuration should follow the exact router, ISP and device documentation. Avoid exposing administrative interfaces directly to the public internet.

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 6