| O. | ONOCRA Home Internet & Wi-Fi · Technology guide |
| --- | --- |

# Understanding Cable Broadband / Coax (HFC)

HFC architecture · DOCSIS 3.0 · DOCSIS 3.1 · DOCSIS 4.0 horizon

Purpose. This guide explains how cable broadband usually uses a hybrid fibre-coax (HFC) network, how DOCSIS generations change capability, why local node load and upstream design matter, what equipment you may see at home, and how cable networks fit alongside fibre in the United States, United Kingdom and Germany. It is a technology guide, not a provider ranking.

Important distinction: cable broadband is usually delivered over hybrid fibre-coax (HFC), not “all coax” from the provider core to the home. Fibre normally runs deep into the network; coaxial cable completes the local access path. DOCSIS is the broadband standard family carried over that HFC plant.

## Evidence key

| Provider / network claim | What the provider, network operator or manufacturer says. Useful, but verify it for the address and plan. |
| --- | --- |
| Independent / official evidence | Regulator, standards body or third-party measurements. Check date, geography and method. |
| ONOCRA editorial interpretation | ONOCRA’s evidence-based explanation or household-fit interpretation. Never presented as a measurement. |
| Unverified / variable | No reliable current evidence, or the answer varies by address/provider. We say so rather than guess. |

## Plain-English summary

| Question | ONOCRA explanation |
| --- | --- |
| What it is | A fixed-line broadband family that typically uses a hybrid fibre-coax (HFC) network: fibre reaches a local optical node and coaxial cable carries the final access segment to the premises. |
| What determines performance | DOCSIS generation, local node/segment load, upstream/downstream spectrum design, modem compatibility, coax condition, provider service tier and upstream network capacity. |
| Why households still use it | Large existing footprints, high downstream capacity, mature installation processes and the ability to upgrade capacity over existing HFC infrastructure. |
| Main limitation | Capacity is shared across a local cable segment/node, and many current cable services remain more constrained upstream than downstream. Poor coax, connectors or splitters can also affect signal quality. |
| Lifecycle | Current/mature access family in the US, UK and Germany. DOCSIS 3.1 is widely deployed; DOCSIS 4.0 is an upgrade generation with selective or emerging deployment rather than a separate access family. |

ONOCRA · Independent, evidence-first technology guidance for households                                                                      Page 1

| O. | ONOCRA Home Internet & Wi-Fi · Technology guide |
| --- | --- |

# 1 · Architecture and DOCSIS generations

| Variant / layer | What it means at home |
| --- | --- |
| HFC architecture | Hybrid fibre-coax: fibre runs from the operator network toward neighbourhood optical nodes; coax carries the final access segment to homes through a cable modem or gateway. |
| DOCSIS 3.0 | An older but still encountered cable broadband generation. Capability depends heavily on channel bonding, spectrum allocation and provider configuration. |
| DOCSIS 3.1 | The current mainstream high-capacity DOCSIS generation in many markets. It uses OFDM/OFDMA and can support gigabit-class downstream service; real upload and download tiers depend on operator deployment. |
| DOCSIS 4.0 | The next major HFC upgrade generation. CableLabs specifies substantially higher upstream capacity and options for more symmetrical multi-gigabit service through Full Duplex DOCSIS and/or Extended Spectrum DOCSIS. Household availability remains operator- and address-specific. |
| Fibre-deep / distributed access | Operators may move fibre and digital network functions closer to customers, split nodes or use Remote PHY/other distributed access designs to increase capacity without immediately replacing every coax drop with FTTP. |

## The shared HFC path matters

| Factor | Why it matters |
| --- | --- |
| Shared node / segment | Households on the same local HFC segment share capacity. Busy-period performance depends on node size, service demand, spectrum allocation and provider engineering. |
| Upstream / downstream spectrum | Cable networks divide spectrum between downstream and upstream traffic. The split and DOCSIS generation strongly affect upload capability. |
| Coax condition and splitters | Damaged coax, poor connectors, excessive splitters, ingress/noise or old in-home wiring can reduce signal quality and reliability. |
| Modem / gateway generation | The modem must support the provider’s DOCSIS generation and service tier. An older or incompatible modem can become the bottleneck even when the network has been upgraded. |
| Powered outside plant and backhaul | HFC nodes, amplifiers and operator equipment depend on power and backhaul/core capacity. A healthy coax drop does not prevent outages or congestion elsewhere in the network. |

## Typical equipment

At the home: a cable modem or provider gateway connected to a coaxial outlet, plus a separate Wi-Fi router if the modem/gateway is used in bridge or modem-only mode where supported. In the outside network, fibre normally reaches a local optical node and coax serves the final access segment to premises.

ONOCRA · Independent, evidence-first technology guidance for households                                                                      Page 2

| O. | ONOCRA Home Internet & Wi-Fi · Technology guide |
| --- | --- |

# 2 · Performance and household use

| Performance factor | What to expect |
| --- | --- |
| Download capacity | Cable broadband can provide high to very high download tiers, particularly on DOCSIS 3.1 networks. The purchased plan, modem and local network design determine the real household result. |
| Upload capacity | Often lower than download on current HFC services. Upstream capability depends on the return-path spectrum, DOCSIS generation, local plant upgrades and provider tier. |
| Symmetry | Many cable plans remain asymmetrical. DOCSIS 4.0 is designed to enable much stronger upstream capacity and can support more symmetrical multi-gigabit services, but this is not proof that a specific address receives such a tier. |
| Latency & jitter | Can be low and suitable for real-time use. Local congestion, signal problems, provider routing and queueing can increase delay or variation. |
| Peak-time consistency | A key HFC consideration because the local access segment is shared. Evening performance may differ if the node is busy or insufficiently provisioned. |
| Signal quality | Coax itself can carry high capacity, but poor connectors, ingress/noise, damaged cable or splitters may cause errors, modem channel issues or instability. |

## Household-fit guide

| Use case | Practical interpretation |
| --- | --- |
| Light browsing, email, messaging | Usually easily handled when the line is healthy and the service tier is adequate. |
| HD / 4K streaming | A common cable strength because downstream capacity can be high. Multiple simultaneous streams still depend on the chosen tier and local congestion. |
| Remote work and video calls | Usually workable, but frequent large uploads or several simultaneous calls make the stated upload rate, latency and jitter important. |
| Cloud backup / creator uploads | Can be much slower than downloads on asymmetrical cable plans. Check the actual upload tier before relying on it for large backups or media workflows. |
| Gaming | Often performs well on a healthy network. Test latency/jitter during busy hours and separate Wi-Fi issues from the cable access connection. |
| Many devices / busy household | High downstream capacity can serve many devices well, but upload-heavy activity and node congestion can still become bottlenecks. |

Do not confuse Wi-Fi speed with cable-broadband speed. A Wi-Fi 6/6E/7 router can improve the local wireless link but cannot exceed the internet tier or fix congestion in the HFC access network. Test by Ethernet when possible to separate Wi-Fi limits from cable access performance.

ONOCRA · Independent, evidence-first technology guidance for households                                                                      Page 3

| O. | ONOCRA Home Internet & Wi-Fi · Technology guide |
| --- | --- |

# 3 · Availability and regional position — US / UK / Germany

| Market | ONOCRA status | What that means | Verify with |
| --- | --- | --- | --- |
| United States | Mature / current | The FCC Broadband Data Collection classifies Coaxial Cable / HFC as fixed technology code 40 and the National Broadband Map shows address-level cable availability reported by providers. DOCSIS generation and plan capability still vary by operator and location. | FCC National Broadband Map / BDC technology code 40 (Coaxial Cable / HFC). |
| United Kingdom | Mature / current | Cable/HFC remains part of the UK fixed-broadband landscape alongside expanding full fibre. Ofcom’s 2026 telecoms review describes DOCSIS over HFC and notes gigabit service using DOCSIS 3.1; full-fibre coverage continues to expand, so the long-term access mix is changing. | Ofcom Connected Nations + provider address checker; for HFC architecture/DOCSIS context use Ofcom Telecoms Access Review 2026-31. |
| Germany | Mature / current alongside fibre | Bundesnetzagentur states that current fixed demand is met by a mix of VDSL copper, HFC cable and FTTH/FTTB fibre. HFC therefore remains a live competing infrastructure while fibre rollout and migration progress region by region. | Bundesnetzagentur Gigabit-Grundbuch/Breitbandatlas + provider address checker. |

## Availability rule for ONOCRA

Always check the exact service address. A country-level cable/HFC label only says the access family exists in that market. It does not prove that a particular provider, DOCSIS generation, upload profile or speed tier is available at a household. For Step 4, ONOCRA should show “verify at your address” unless location-specific evidence is available.

## Installation and practical checks

| Scenario | What to check |
| --- | --- |
| Existing active cable outlet | Often the simplest case, but confirm the outlet is connected to the current provider network and that the modem/gateway is compatible with the service tier. |
| New or inactive property | May require an engineer visit, outside-drop work, outlet replacement or activation. Do not assume a visible coax socket is serviceable. |
| Renters / apartments | Building distribution, landlord or management-company rules can matter. Ask whether the provider can use existing internal coax or needs new cabling. |
| Using your own equipment | Some providers allow approved customer-owned modems/routers; others require their gateway for authentication, voice or support. Check the approved-device list and bridge-mode policy. |
| Fault support | Ask who maintains the drop cable, internal coax, wall outlet and modem; what signal tests are available; and what repair targets or compensation rules apply. |

ONOCRA · Independent, evidence-first technology guidance for households                                                                      Page 4

| O. | ONOCRA Home Internet & Wi-Fi · Technology guide |
| --- | --- |

# 4 · Cost, contract, lifecycle and upgrade decisions

| Commercial / lifecycle factor | ONOCRA guidance |
| --- | --- |
| Monthly price | Compare the regular price after introductory discounts and bundles. Cable is not automatically cheaper than fibre or FWA; total value is market- and provider-specific. |
| Setup / equipment | Check activation, engineer, modem/gateway rental or purchase, delivery, return and replacement fees. |
| Contract term / exit | Read minimum term, annual price-change rules, cancellation fees and what happens if the provider migrates the property to fibre or another network architecture. |
| Data / traffic policy | Many fixed cable plans are unlimited, but caps, fair-use, congestion-management or prioritisation rules can exist. Verify the specific plan. |
| Upgrade path | HFC can evolve through node splits, fibre-deep/distributed access, higher spectrum, modem upgrades and DOCSIS 4.0. These are operator investments; a DOCSIS specification does not guarantee an address-level upgrade date. |
| Migration / replacement risk | Some operators may continue upgrading HFC while others move progressively to FTTP. Check the provider’s local roadmap before treating “cable” as either obsolete or permanently fixed in place. |

## Questions to ask before you sign

1. Is this HFC cable broadband, and which DOCSIS generation serves the address?

2. What download/upload figures are advertised, typical, minimum or guaranteed?

3. Which modem/gateway is supplied, and is it compatible with the service tier?

4. What does the provider say about evening performance, node load or upstream limits?

5. What is the regular price after promotions, plus setup, equipment and exit costs?

6. Are there data caps, traffic-management, fair-use or upload restrictions?

7. Can I use my own modem/router or bridge mode, and what compatibility rules apply?

8. What local upgrade is planned: DOCSIS 3.1/4.0, node split, fibre-deep or FTTP?

## Step 4 extraction — canonical fields

| Field group | Cable Broadband / Coax (HFC) values |
| --- | --- |
| architecture | HFC; fibre to local optical node; coax final access segment; shared node/segment |
| performance | download/upload profile; symmetry; latency/jitter; peak-time consistency; signal quality |
| installation | cable modem/gateway; coax outlet/drop; engineer need; landlord/building constraints |
| commercial | price snapshot; activation/equipment/rental fees; contract; data/traffic policy |
| networking | modem/gateway mode; router choice; public IP/IPv6/port forwarding where relevant |
| lifecycle | DOCSIS generation; node/fibre-deep roadmap; DOCSIS 4.0 status; possible FTTP migration |
| evidence | source URL/date; address-specific availability; provider tier; modem generation; confidence; what to verify |

ONOCRA · Independent, evidence-first technology guidance for households                                                                      Page 5

| O. | ONOCRA Home Internet & Wi-Fi · Technology guide |
| --- | --- |

# 5 · Glossary and verification sources

| Term | Plain meaning |
| --- | --- |
| HFC | Hybrid fibre-coax: fibre runs deep into the operator network; coax carries the final access path to premises. |
| DOCSIS | Data Over Cable Service Interface Specification: the standard family used for broadband over cable/HFC. |
| DOCSIS 3.1 | A mainstream modern DOCSIS generation using OFDM/OFDMA and supporting high-capacity cable broadband. |
| DOCSIS 4.0 | The next major DOCSIS generation, designed for much stronger upstream and more symmetrical multi-gigabit service. |
| Node | A local point where fibre feeds the coax access segment; capacity is shared among premises on that node/segment. |
| Downstream / upstream | Traffic from network to home / home to network. Cable often allocates more capacity downstream than upstream. |
| OFDM / OFDMA | Multi-carrier modulation/access techniques used by DOCSIS 3.1+ to use spectrum efficiently. |
| Cable modem / gateway | The home device linking coax broadband to Ethernet and, in a gateway, routing and often Wi-Fi. |

## Regional and standards verification sources — checked 20 October 2026

| Source | What ONOCRA uses it for | URL |
| --- | --- | --- |
| US · FCC Broadband Data Collection | Coaxial Cable / HFC is fixed technology code 40; the National Broadband Map supports address-level cable availability and provider-reported maximum advertised speeds. | Open official source |
| UK · Ofcom Telecoms Access Review 2026-31 | Explains the HFC architecture and DOCSIS broadband layer; notes DOCSIS 3.1 gigabit service and the role of DOCSIS generation/network configuration in capability. | Open official source |
| Germany · Bundesnetzagentur | States that current fixed demand is met by a mix of VDSL, HFC and FTTH/FTTB while fibre rollout continues region by region. | Open official source |
| CableLabs · DOCSIS 4.0 Technology | Defines DOCSIS 4.0 as an HFC upgrade generation with substantially increased upstream capacity and Full Duplex / Extended Spectrum options. Specification capability is not the same as a household plan result. | Open official source |

Publication rule. This document is general information, not a laboratory test, provider recommendation or guarantee of availability. Verify the current access technology, DOCSIS generation, modem compatibility, speeds, price, contract terms, traffic policy, installation requirements and upgrade status for the exact address before choosing or changing service.

ONOCRA · Independent, evidence-first technology guidance for households                                                                      Page 6