| O. | ONOCRA Home Internet & Wi-Fi · Technology guide |
| --- | --- |

Understanding Full Fibre / FTTP

FTTP vs FTTB/FTTC · GPON · XGS-PON · ONT · shared PON capacity · symmetry · installation · upgrade path

Purpose. This guide explains what “full fibre” really means, how fibre-to-the-premises (FTTP) differs from fibre-to-the-building (FTTB) and fibre-to-the-cabinet (FTTC), how passive optical network (PON) generations shape capability, and what a household should verify before choosing or upgrading a service.

Important distinction: FTTP means the optical fibre access path reaches the premises. FTTB can stop at the building and use another medium inside; FTTC stops at a cabinet and normally uses copper for the final segment. “Fibre broadband” in marketing should therefore be checked against the actual last-mile architecture.

Evidence key

| Evidence label | How ONOCRA uses it |
| --- | --- |
| Provider / network claim | What the provider, network operator or equipment maker says. Useful, but verify the exact address, plan and equipment. |
| Independent / official evidence | Regulator, standards body or third-party measurements. Check date, geography and method. |
| ONOCRA editorial interpretation | ONOCRA’s evidence-based explanation or household-fit interpretation. Never presented as a measurement. |
| Unverified / variable | No reliable current evidence, or the answer varies by address/provider. We say so rather than guess. |

Plain-English summary

| Question | ONOCRA explanation |
| --- | --- |
| What it is | A fixed access family in which optical fibre reaches the home or premises and terminates at an optical network terminal (ONT) or equivalent optical network unit. |
| What determines performance | The PON or point-to-point architecture, optical split/capacity, provider provisioning, service tier, backhaul/core network, ONT/router capability and local Wi-Fi. |
| Why households choose it | High capacity, strong upload potential, low and usually steady access latency, and an upgrade path that can reuse the fibre while electronics evolve. |
| Main limitation | Availability is address-specific and installation may require new fibre routing, drilling, building permission or internal cabling. A fibre line also does not guarantee good Wi-Fi. |
| Lifecycle | Current/mature access family in the US, UK and Germany. GPON remains common; XGS-PON and later PON generations extend capacity without making FTTP a different access family. |

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 1

| O. | ONOCRA Home Internet & Wi-Fi · Technology guide |
| --- | --- |

1 · Architecture, fibre boundaries and PON generations

| Variant / layer | What it means at home |
| --- | --- |
| FTTP / FTTH | Fibre reaches the individual premises/home and terminates at an ONT/ONU. This is the canonical ONOCRA “Full Fibre / FTTP” family. |
| FTTB | Fibre reaches the building, but the path from the building distribution point to an individual flat/unit may use Ethernet, coax, copper or another medium. Do not automatically label every FTTB service as FTTP. |
| FTTC | Fibre reaches a street cabinet or curb-side point; copper usually completes the final access segment. This belongs under DSL/Copper Broadband when copper remains the last mile. |
| GPON | A widely deployed passive optical network generation. ITU-T G.984 describes GPON with an OLT in the provider network, passive optical distribution, and ONT/ONU at customer premises. |
| XGS-PON | A 10-Gigabit-capable symmetric PON generation standardised in ITU-T G.9807.1. It provides nominal 10 Gbit/s line rates in both downstream and upstream directions, but a household plan is not automatically 10 Gbit/s. |
| Later PON evolution | 25G/50G-class and other advanced PON systems belong under the same FTTP access family. ONOCRA should surface them as generation/upgrade context only when provider and address evidence supports them. |

Shared PON capacity: “full fibre” does not necessarily mean a dedicated fibre capacity pool

| Factor | Why it matters |
| --- | --- |
| OLT and optical distribution | An optical line terminal (OLT) in the provider network serves customer optical network terminals through an optical distribution network. |
| Passive splitters | Many residential PON designs use passive splitters so optical capacity is shared among a group of premises. Split ratios and engineering vary by provider. |
| Provisioning and service tier | The line technology defines technical potential; the purchased plan, provider policies and network provisioning define the service delivered to the household. |
| Backhaul / core network | A healthy optical access link can still be limited by provider backhaul, routing, service incidents or wider network congestion. |
| Power dependency | The fibre itself is passive, but the ONT and home router need power. Provider equipment upstream also needs power and network resilience. |

Typical equipment

At the home: an ONT/ONU or integrated optical gateway, plus a router/Wi-Fi system. Some providers supply separate ONT and router devices; others use an integrated gateway. Check who owns the ONT, whether bridge/router choices are supported, and what must be returned after cancellation.

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 2

| O. | ONOCRA Home Internet & Wi-Fi · Technology guide |
| --- | --- |

2 · Performance and household use

| Performance factor | What to expect |
| --- | --- |
| Download capacity | FTTP can support very high service tiers. The available tier depends on the PON generation, optical design, provider provisioning and commercial plan. |
| Upload capacity | FTTP can provide strong upload capacity. Some plans are symmetrical; others are deliberately provisioned with lower upload than download despite the optical access medium. |
| Symmetry | Do not assume “fibre” means symmetrical. XGS-PON is a symmetric line-rate technology, but retail plans can still be asymmetrical. Verify the stated plan upload figure. |
| Latency & jitter | The optical access path is generally well suited to low and steady latency, but end-to-end results still depend on routing, congestion, equipment and the destination service. |
| Peak-time consistency | Often strong on a well-provisioned network. PON capacity is still shared in many deployments, so the network design and provider capacity planning matter. |
| Signal medium | Optical fibre is immune to electromagnetic interference on the access line. Service can still fail because of physical fibre damage, dirty/poor optical connections, equipment faults, power loss or wider network issues. |

Household-fit guide

| Use case | Practical interpretation |
| --- | --- |
| Light browsing, email, messaging | Easily handled on normal FTTP tiers; a very high headline speed is rarely necessary for this use alone. |
| HD / 4K streaming | Strong fit when the plan has enough capacity for simultaneous streams. Local Wi-Fi quality still determines the device experience. |
| Remote work and video calls | Strong upload potential and steady latency can help multiple simultaneous calls, screen sharing and large transfers. |
| Cloud backup / content creation | A major reason to check symmetrical or high-upload fibre tiers. Upload-heavy households can benefit substantially compared with many asymmetrical alternatives. |
| Gaming | Low and steady access latency is helpful, but game server location, routing and local Wi-Fi/Ethernet matter. Fibre does not eliminate all latency. |
| Many devices / busy household | FTTP provides a strong WAN foundation, but the router, Wi-Fi generation, access-point placement and wired backhaul still determine distribution inside the home. |

Do not confuse FTTP with Wi-Fi. FTTP describes the provider access path to the premises. Wi-Fi 5/6/6E/7 describes the local wireless network inside the home. A fast fibre plan can still feel slow in rooms with poor Wi-Fi coverage.

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 3

| O. | ONOCRA Home Internet & Wi-Fi · Technology guide |
| --- | --- |

3 · Availability and regional position — US / UK / Germany

| Market | ONOCRA status | What that means | Verify with |
| --- | --- | --- | --- |
| United States | Mature / current | The FCC Broadband Data Collection uses technology code 50 for Optical Carrier / Fiber to the Premises and explicitly excludes fibre-to-the-curb from that category. Availability is location-specific. | FCC National Broadband Map / BDC technology code 50 + provider address checker. |
| United Kingdom | Mature / rapidly expanding | Ofcom’s Spring 2026 snapshot reported full fibre available to 24.9 million UK residential premises (82%) as of January 2026, with further build planned. | Ofcom Connected Nations + provider/Openreach or alternative-network address checker. |
| Germany | Mature / expanding alongside legacy access | The Breitbandatlas tracks fixed broadband by technology and provides current coverage data. FTTH/FTTB deployment is expanding while DSL/VDSL and HFC remain part of the market. | Gigabit-Grundbuch / Breitbandatlas + provider address checker; distinguish FTTH/FTTB from FTTC/VDSL. |

Availability rule for ONOCRA

Always check the exact service address. Country-level “full fibre available” statistics show market maturity, not whether a particular home can order FTTP from a provider. For flats and multi-dwelling buildings, also verify whether fibre reaches the individual dwelling or only the building.

Installation and practical checks

| Scenario | What to check |
| --- | --- |
| Existing FTTP property | Confirm the ONT is active/compatible, which provider/network owns it, whether a new router is needed, and whether the ordered plan can be activated remotely. |
| New fibre installation | Ask where fibre will enter, whether drilling/external boxes are needed, where the ONT will sit, cable-length limits, power-socket requirements and whether an engineer visit is required. |
| Renters / apartments | Check landlord, building-owner or management permission and whether the building’s internal distribution reaches the individual unit. |
| Router placement | The ONT location may not be the best Wi-Fi location. Plan Ethernet from the ONT/router to a central access point or mesh backhaul where practical. |
| Fault support | Ask who is responsible for the external fibre, internal fibre lead, ONT and router; what optical/alarm indicators should be recorded; and what the repair commitment is. |

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 4

| O. | ONOCRA Home Internet & Wi-Fi · Technology guide |
| --- | --- |

4 · Cost, contract, lifecycle and upgrade decisions

| Commercial / lifecycle factor | ONOCRA guidance |
| --- | --- |
| Monthly price | Compare the regular price after introductory discounts and bundles. FTTP is not automatically the cheapest or most expensive option; compare total household value. |
| Setup / equipment | Check installation/activation fees, ONT/router charges, delivery, engineer work, replacement policy and equipment-return terms. |
| Contract term / exit | Read minimum term, annual price-change rules, cancellation/early-exit fees, and whether moving home changes the contract. |
| Data / traffic policy | Many fixed fibre plans are unlimited, but verify any fair-use, traffic-management or provider-specific restrictions instead of assuming. |
| Upgrade path | A major strength is that the optical distribution fibre can often remain while OLT/ONT electronics and service tiers evolve from GPON toward XGS-PON or later generations. |
| Migration / replacement risk | FTTP is a long-lived access architecture, but providers may change electronics, router requirements, wholesale network arrangements or commercial plans. Treat “future-proof” as an editorial tendency, not a guarantee. |

Questions to ask before you sign

1. Does fibre actually reach my premises/dwelling, or does it stop at the building/cabinet?

2. Which access/PON generation serves the address (for example GPON or XGS-PON), and what service tier is offered?

3. What are the stated download and upload speeds, and is the plan symmetrical or asymmetrical?

4. Where will the fibre and ONT be installed, and what drilling, landlord/building permission or power requirements apply?

5. What are the regular price, setup/equipment fees, contract term, annual price changes and exit charges?

6. Can I use my own router or bridge mode, and what public-IP/IPv6/port-forwarding options are available if I need them?

7. What happens during a power cut, and which ONT/router backup options are supported or practical?

8. What network/equipment upgrade path is planned for this address, and will future higher tiers require a new ONT/router?

Step 4 extraction — canonical fields

| Field group | Full Fibre / FTTP values |
| --- | --- |
| architecture | FTTP/FTTH; fibre reaches premises; distinguish FTTB/FTTC; PON or point-to-point; OLT/ODN/ONT; shared split/capacity where relevant |
| performance | download/upload profile; symmetry; latency/jitter; peak-time consistency; plan tier; provider provisioning |
| installation | ONT/gateway; fibre entry/route; engineer need; power socket; landlord/building constraints; Ethernet/Wi-Fi handoff |
| commercial | price snapshot; installation/equipment fees; contract; exit terms; data/traffic policy |
| networking | router/bridge mode; public IP; CGNAT where relevant; IPv6; port forwarding; provider authentication |
| resilience | ONT/router power dependency; provider outage behaviour; backup options; physical fibre damage/fault process |
| lifecycle | GPON/XGS-PON generation; OLT/ONT upgrade path; 25G/50G-class horizon; provider-specific migration/wholesale changes |
| evidence | source URL/date; exact-address availability; provider plan/tier; PON generation if disclosed; confidence; what to verify |

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 5

| O. | ONOCRA Home Internet & Wi-Fi · Technology guide |
| --- | --- |

5 · Glossary and verification sources

| Term | Plain meaning |
| --- | --- |
| FTTP / FTTH | Fibre to the premises / home: optical fibre reaches the customer premises and terminates there. |
| FTTB | Fibre to the building: fibre reaches the building, but another medium may serve individual units. |
| FTTC | Fibre to the cabinet/curb: fibre stops before the premises and another medium, commonly copper DSL, completes the last segment. |
| PON | Passive optical network: point-to-multipoint optical access using passive splitters between provider OLT and customer ONT/ONU. |
| GPON | Gigabit-capable PON defined by the ITU-T G.984 family; a widely deployed residential fibre-access generation. |
| XGS-PON | 10-Gigabit-capable symmetric PON defined in ITU-T G.9807.1, with nominal 10 Gbit/s downstream and upstream line rates. |
| OLT | Optical line terminal: provider-side equipment that serves one or more optical distribution networks. |
| ONT / ONU | Optical network terminal/unit: the customer-side optical termination that converts the access link for home equipment. |
| Symmetrical | Upload and download service speeds are the same or very similar. Do not infer this from “full fibre” alone. |
| Optical split | A passive division of an optical signal among multiple premises in many PON deployments; split ratio and capacity engineering vary. |

Regional and standards verification sources — checked 20 October 2026

| Source | What ONOCRA uses it for | Link |
| --- | --- | --- |
| US · FCC Broadband Data Collection | Defines technology code 50 as Optical Carrier / Fiber to the Premises; excludes fibre-to-the-curb and supports location-level verification. | Open official source |
| UK · Ofcom Connected Nations Spring 2026 | Snapshot of UK full-fibre availability and take-up as of January 2026; use with provider/address checkers. | Open official source |
| Germany · Gigabit-Grundbuch / Breitbandatlas | Official fixed-broadband technology coverage data and documentation; distinguish FTTH/FTTB from FTTC/VDSL. | Open official source |
| ITU-T · G.984.1 GPON | General characteristics of GPON, including OLT/ONT and passive optical distribution architecture. | Open official source |
| ITU-T · G.9807.1 XGS-PON | In-force XGS-PON recommendation; 10-Gigabit-capable symmetric PON with nominal 10 Gbit/s line rates both directions. | Open official source |

Publication rule. This document is general information, not a laboratory test or provider recommendation. Verify exact-address FTTP availability, plan speeds, pricing, contract terms, installation requirements, PON/equipment generation where disclosed, and current regional status before making a buying or switching decision.

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 6