| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-06 What Happens in a Power Cut? ONT, Modem, Router, FWA & Satellite Home Internet & Wi-Fi · Decision & evidence guide |
| --- | --- |

Purpose. This guide explains why home internet can stop during a power cut even when the provider's wider network is still operating - and why powering only the Wi-Fi router may not be enough. It separates in-home equipment power from access-network resilience across fibre, cable, DSL, fixed wireless and satellite.

ONOCRA rule: Never promise that internet will remain available in a power cut merely because a router, ONT or terminal is on battery. Every required device in the home must stay powered, and the provider's upstream network must also remain operational.

## The seven power-cut terms households should understand

| Term | Plain meaning | Why it matters |
| --- | --- | --- |
| Customer-premises equipment (CPE) | The active equipment installed at the home, such as an ONT, modem, gateway, router, mesh node, switch or phone adaptor. | Any required powered device that shuts down can break the connection. |
| ONT / fibre service box | The powered device that terminates a fibre connection at or inside the home. | Full-fibre service normally depends on the ONT, or an integrated optical gateway, having power. |
| Modem / gateway | The device that links the home to DSL, cable or another provider access network; some gateways also include routing and Wi-Fi. | Backing up a separate router does not help if the modem or gateway has lost power. |
| Router / Wi-Fi / mesh | The in-home network that shares the internet connection with wired and wireless devices. | The access line may still be live while the household appears offline because the router or mesh system is unpowered. |
| UPS / battery backup | A local power source that can keep selected equipment running when mains electricity fails. | Runtime depends on usable battery energy, device load, conversion losses, battery age and temperature. |
| Provider-side backup power | Batteries, generators or other resilience used in the provider's exchange, cabinet, node, headend, mobile site or other network facilities. | Home backup cannot restore service if the provider-side path is down. |
| Alternative access path | A second way online, such as mobile data, a second ISP or another independent connection. | For important continuity needs, diversity can matter as much as battery runtime. |

## The key distinction

A power cut has at least two separate layers: power inside the home and power in the provider network. A UPS can solve the first layer only for the equipment connected to it. It cannot guarantee that a street cabinet, cable node, mobile site, exchange, satellite ground system or other upstream facility will remain available for the whole outage.

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 1

| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-06 1 · Trace the power path before an outage Home Internet & Wi-Fi · Decision & evidence guide |
| --- | --- |

## A safe household audit sequence

| Step | Check | Interpretation / caution |
| --- | --- | --- |
| 1 | Identify the access type: DSL/FTTC, cable/HFC, full fibre/FTTP, fixed wireless, 4G/5G FWA or satellite. | Do not infer power-cut behaviour from the technology name alone; provider network design and local equipment differ. |
| 2 | List every powered device between the provider line/terminal and the devices you actually use: ONT, modem/gateway, router, mesh nodes, switches, access points and phone adaptors. | One unpowered dependency can interrupt the whole chain. |
| 3 | Record each device model and its power adapter rating; note whether the ONT is separate or integrated into the gateway. | Use manufacturer/provider information where available. Adapter maximum ratings are not the same as measured average power draw. |
| 4 | Check any digital landline, cordless phone base, alarm, telecare or other service that depends on broadband equipment. | The internet backup plan and the emergency-calling plan may require different equipment and should be verified separately. |
| 5 | Check whether any provider-supplied battery backs up internet data, voice only, or both. | Some backup systems preserve basic voice service but not full internet use. Read the exact provider documentation for the installed equipment. |
| 6 | Test the backup arrangement using the UPS/device manufacturer method or a controlled short mains interruption when it is safe to do so. | Do not deliberately interrupt equipment used for medical, telecare, alarm or other critical functions. Avoid repeated hard power cycles or factory resets. |
| 7 | Save the provider outage page/support contact and decide the fallback if the fixed connection fails upstream. | A charged phone, mobile hotspot, second provider or other independent path can be more useful than extending one local battery indefinitely. |

## Recognise the household power layers

| Device / layer | Usually needs local power? | What a household outage can do |
| --- | --- | --- |
| ONT / fibre service box | Yes | If unpowered, fibre service to the home normally stops even if the fibre network outside is still operating. |
| DSL or cable modem / gateway | Yes | The access signal may exist outside, but the powered modem/gateway is still needed to use it. |
| Router / Wi-Fi / mesh controller | Yes | Internet may be available at the modem/ONT while household Wi-Fi and routing disappear. |
| Mesh nodes / access points / Ethernet switches | Yes, unless powered from another backed-up source | Coverage or wired segments can fail even when the main router remains online. |
| 4G / 5G FWA gateway | Yes | The gateway needs alternative power; the serving mobile network must also remain available. |
| Satellite terminal + router / power supply | Yes | The terminal and networking equipment require power; power draw can be materially higher than a simple Wi-Fi router. |

Safety safeguard: use only compatible UPS/battery equipment and follow the device, battery and electrical-safety instructions. Do not improvise power connections to provider equipment. Never run fuel-burning generators indoors or in enclosed spaces.

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 2

| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-06 2 · What a power cut can change by access technology Home Internet & Wi-Fi · Decision & evidence guide |
| --- | --- |

## The access network and the home power chain are separate

| Access / service | What the household must usually keep powered | What can still fail upstream |
| --- | --- | --- |
| DSL / copper / FTTC | Modem/router or provider gateway; any separate Wi-Fi/mesh equipment. | Exchange/cabinet electronics, backhaul or wider provider network can fail or exhaust backup. |
| Cable / HFC | Cable modem/gateway plus router/mesh if separate. | Neighbourhood optical/coax nodes, amplifiers, headend or backhaul depend on provider power/resilience design. |
| Full fibre / FTTP | ONT or integrated fibre gateway plus router/mesh and any required switches. | Passive fibre itself does not need local street power, but active provider equipment elsewhere still does. |
| Fixed wireless (non-mobile) | Outdoor/indoor radio CPE, PoE injector where used, and router/Wi-Fi equipment. | Provider base station, relay/backhaul and site backup determine whether the radio path remains available. |
| 4G / 5G FWA | Home internet gateway/router and any external receiver if used. | The serving mobile site and transport network may have backup, but duration and capacity are provider- and site-specific. |
| Satellite | User terminal/dish, power supply and router or integrated Wi-Fi equipment. | Service also depends on the satellite system, gateway/backhaul and account/service availability; local battery power is only one part. |
| Digital / VoIP landline over broadband | All equipment required for the voice path, which may include ONT, gateway/router, phone adaptor and the telephone itself. | A provider voice battery may be limited to voice service; cordless phone bases also need power. |
| Mobile hotspot / phone tethering fallback | Charged phone/hotspot and any device being connected. | Local mobile network availability, congestion and site power still determine whether the fallback works. |

## What a UPS can and cannot solve

| Situation | A UPS may help | Do not assume |
| --- | --- | --- |
| Short household power cut | Keeps connected CPE running if capacity and load are sufficient. | That every required box is actually connected to the UPS. |
| Brief voltage dip / restart cycle | Can prevent needless router, modem or ONT rebooting. | That a UPS protects against every electrical event; use appropriate surge/electrical protection. |
| Router is backed up but ONT/modem is not | Only the router remains powered. | That Wi-Fi lights mean the internet path is alive. |
| Provider-side network outage | Local equipment can stay powered and ready. | That local battery power can restore an upstream node, mobile site, cabinet, headend or provider backhaul. |
| Long outage / battery exhausted | Runtime can be extended with a larger compatible power source or recharging plan. | That advertised battery capacity equals usable runtime under real load. |

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 3

| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-06 3 · Backup power planning without false promises Home Internet & Wi-Fi · Decision & evidence guide |
| --- | --- |

## Choose the backup design around the whole connection chain

| Option | When it can fit | Main safeguards / limitations |
| --- | --- | --- |
| UPS for router only | The modem/ONT is already independently backed up or integrated into the same powered gateway. | Cheap and simple, but useless if another required access device loses power. |
| UPS for ONT/modem + router | Fixed broadband where separate access and routing devices must stay alive. | Size for the combined load, power adapters and desired runtime; include switches/PoE devices only if essential. |
| Provider-approved battery unit | Provider equipment supports a documented battery accessory. | Confirm whether it backs up internet, voice only or selected functions; maintenance/replacement responsibility may fall on the customer. |
| Home battery / inverter system | The household already has resilient AC power and wants to support networking with other essential loads. | Networking runtime depends on the larger household load and inverter/battery state, not only the router wattage. |
| Generator | Long outages where safe, properly installed generation is justified. | Use correct electrical arrangements and outdoor ventilation. A generator does not guarantee provider-network availability. |
| Powered 4G / 5G backup gateway | Fixed broadband fails but the local mobile network remains available. | The gateway itself needs power; performance and availability can fall during area-wide outages or congestion. |
| Phone hotspot / portable hotspot | Short-term fallback for a few important devices. | Battery, signal, data allowance, thermal load and mobile-network availability limit usefulness. |
| Satellite backup | A genuinely independent path is valuable and the terminal can be powered safely. | Terminal power demand can be significant; verify exact hardware requirements and do not assume severe weather or provider outages are irrelevant. |

## Runtime: calculate a planning range, not a promise

A rough starting point is usable battery watt-hours divided by the combined equipment watts. Real runtime will be lower or variable because of inverter/UPS losses, battery reserve limits, battery age, temperature and changing device load. Use measured or manufacturer-supported power figures where possible and keep a margin rather than publishing a single guaranteed runtime.

## If internet fails during a power cut

• Confirm that every required device in the chain is still powered - not only the Wi-Fi router.

• Check ONT/modem/gateway status lights and local device links before rebooting anything.

• Use mobile data to check the provider's outage page or status message if available.

• If the home equipment is powered but the WAN/service light is down, treat an upstream/provider outage as a leading possibility.

• Avoid repeated power cycles or factory resets during an area outage; move to the planned fallback path if continuity matters.

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 4

| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-06 4 · Regional and provider verification - US / UK / Germany Home Internet & Wi-Fi · Decision & evidence guide |
| --- | --- |

There is no ONOCRA country-wide promise for how long home broadband will survive a power cut. Customer-premises equipment, provider backup design, access technology and voice-service rules differ. Evidence must be captured for the exact provider, equipment and plan, and dated.

| Market | What ONOCRA should verify | Evidence hierarchy |
| --- | --- | --- |
| United States | Which customer-premises devices require backup; whether any provider battery supports voice only or internet too; exact equipment supported; expected runtime basis; mobile/fixed-wireless fallback power needs. | 1) Current provider support/equipment page 2) Installed-equipment documentation 3) Current federal/state requirements where relevant 4) Controlled local test as supporting evidence. |
| United Kingdom | Whether broadband/digital landline needs local mains power; provider backup solution for people who depend on the landline; exact equipment covered; mobile fallback and local signal. | Ofcom consumer guidance + provider documentation. Ofcom says digital landlines need mains power and qualifying customers without a working/reliable mobile alternative must be offered a free solution for emergency calling, with at least one hour of backup. |
| Germany | Which ONT/modem/router devices need household power; provider guidance for power cuts; fixed and mobile fallback options; digital-voice implications; exact plan/equipment support. | Provider equipment/support documentation first. Telekom documentation, for example, warns that fixed-line telephones connected to its Speedport cannot be used during a power cut. Verify the exact provider and hardware. |

## Questions to ask before relying on internet during a power cut

| # | Question |
| --- | --- |
| 1 | Which exact boxes in my home must remain powered for internet to work? |
| 2 | Is the ONT separate from the gateway/router, or integrated into it? |
| 3 | Does the provider-supplied battery keep internet data working, or only voice/emergency calling? |
| 4 | Can I use my own UPS or battery system with the installed equipment without affecting support or warranty? |
| 5 | What is the equipment's normal power draw, and is there any meaningful start-up/peak requirement? |
| 6 | Does the provider publish anything about network-side backup or outage resilience for this service? |
| 7 | For FWA, what happens if the serving mobile site loses commercial power or becomes congested? |
| 8 | For satellite, what exact terminal/router power input and average consumption should I plan around? |
| 9 | If I have a digital landline or telecare/alarm device, what is the provider-supported power-cut solution? |
| 10 | What independent fallback will I use if the provider network fails even though my home equipment is still powered? |

ONOCRA buying rule: If connectivity during outages is essential, both the household power chain and an independent fallback path are suitability gates - not accessories to think about after installation.

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 5

| O. | ONOCRA DECISION & EVIDENCE GUIDE · DEC-06 5 · Step 4 evidence fields, glossary & verification sources Home Internet & Wi-Fi · Decision & evidence guide |
| --- | --- |

## Step 4 structured evidence handoff

| Field group | Standard fields | Why Step 4 needs it |
| --- | --- | --- |
| Home power chain | access_type · ont_present · modem_gateway_present · router_mesh · poe_dependencies · voice_adapter · critical_devices | Shows which local devices must stay powered for the connection and related services to remain usable. |
| Backup capability | backup_type · provider_battery_scope · customer_ups_supported · power_draw_w · battery_wh · runtime_basis · maintenance_owner | Separates a real documented backup design from a vague 'has battery' claim and records what the battery actually powers. |
| Provider/network resilience | provider_power_cut_guidance · access_node_dependency · mobile_site_dependency · satellite_dependency · upstream_outage_note | Prevents ONOCRA from implying that local backup power guarantees end-to-end service. |
| Emergency & fallback | digital_voice_power_cut · emergency_call_solution · mobile_fallback · second_connection · fallback_limitations | Captures whether the household has a workable alternative when the primary service or local power chain fails. |
| Evidence | provider_source_url · equipment_model · observation_date · plan_name · region · confidence | Power-cut behaviour can change with hardware, provider policy and network design, so every claim needs attributable evidence. |

## Terms in plain words

| Term | Meaning |
| --- | --- |
| ONT | Optical Network Terminal: powered customer-premises equipment that terminates a fibre access link. |
| Gateway | Provider or customer device combining some or all modem/ONT, routing, firewall and Wi-Fi functions. |
| UPS | Uninterruptible Power Supply: battery-backed power that can keep connected equipment running during a mains interruption. |
| Watt (W) | Rate of electrical power use. Combined device watts influence how quickly a backup battery is depleted. |
| Watt-hour (Wh) | Unit of stored/used energy. Usable watt-hours divided by load watts provide a rough starting point for runtime planning. |
| PoE | Power over Ethernet: sends electrical power with network data over Ethernet cable to devices such as access points or radios. |
| Digital voice / VoIP | Voice service carried through powered digital network equipment rather than relying on a traditional line-powered analogue path. |
| Network-side backup | Provider batteries, generators or other resilience outside the home; scope and runtime are provider- and site-specific. |

## Standards & verification sources - checked 20 October 2026

Ofcom - digital landlines and power cuts / severe-weather connectivity: Open official source

AT&T - battery backup guidance for gateways, modems and external ONTs: Open official source

Telekom Germany - Speedport Smart 4 Plus manual, power-cut emergency-call warning: Open official source

Starlink - official terminal/router power-use guidance: Open official source

General information only. ONOCRA does not operate a test laboratory and should not publish a power-cut runtime or provider-resilience claim without dated evidence. Follow the exact ISP, equipment, UPS/battery and electrical-safety instructions. Backup power at home does not guarantee that the provider network will remain available.

ONOCRA · Independent, evidence-first technology guidance for households                                      Page 6