Tablet Troubleshooting Matrix

Most common tablet troubleshooting searches by US, UK and German residents, with step-by-step solutions and video references.

Covers iPad, Samsung Galaxy Tab, Amazon Fire and other Android tablets. Back up important data before any reset. Query lists are based on commonly reported faults in consumer support guides, not measured search volumes; German queries are natural-language equivalents. Use the video search phrases to find an official-channel video, then paste its link in the Video link column.

| No. | Issue | Search queries (US / UK / DE) | Step-by-step solution | Video search phrases | Video link |
| --- | --- | --- | --- | --- | --- |
| 1 | Won't turn on / black screen | US: tablet won't turn on UK: tablet not turning on DE: Tablet geht nicht an / Tablet startet nicht | 1. Charge with the original charger and cable for at least 1 hour (overnight if possible). 2. Try a different socket, cable and adapter. 3. While plugged in, force restart: iPad (Face ID/top button) – press Volume Up, Volume Down, then hold Top button until Apple logo; iPad (Home button) – hold Home + Top; Samsung – hold Power + Volume Down ~10 sec; Fire – hold Power ~40 sec; other Android – hold Power 30 sec. 4. Check the charging port for lint or debris. 5. Still dead: contact the maker or an authorised repairer (battery or board fault). | EN: iPad won't turn on force restart ¦ Samsung Galaxy Tab won't turn on fix DE: iPad startet nicht neu starten erzwingen ¦ Samsung Tablet geht nicht an |  |
| 2 | Won't charge / charges slowly | US: tablet won't charge UK: tablet not charging DE: Tablet lädt nicht / lädt langsam | 1. Swap the cable first – worn cables cause most charging faults. 2. Use the maker's adapter or one with enough wattage; avoid computer USB ports. 3. Try another wall socket. 4. Switch off, then gently clean the port with a dry wooden/plastic toothpick or soft brush (never metal). 5. Remove the case; avoid charging while gaming or streaming. 6. Restart while plugged in and update the software. 7. Charges only at certain angles or loses charge fast: port or battery repair. | EN: tablet not charging fix ¦ Galaxy Tab slow charging fix DE: Tablet lädt nicht Lösung ¦ Samsung Tablet lädt langsam |  |
| 3 | Slow / lagging | US: tablet running slow UK: tablet slow and laggy DE: Tablet langsam / Tablet ruckelt | 1. Restart the tablet. 2. Check storage – a nearly full tablet slows down a lot (see No. 9). 3. Close and delete unused apps. 4. Android: clear app cache (Settings > Apps > app > Storage > Clear cache). 5. Install pending system and app updates. 6. iPad: turn off Background App Refresh (Settings > General). 7. Last resort: back up and factory reset (No. 17). On old tablets a worn battery or ageing hardware may be the limit. | EN: how to speed up slow tablet ¦ speed up iPad DE: Tablet schneller machen ¦ iPad langsam was tun |  |
| 4 | Won't connect to Wi-Fi | US: tablet won't connect to wifi UK: tablet wifi keeps dropping DE: Tablet WLAN verbindet nicht | 1. Check whether another device connects; if not, restart the router. 2. Toggle Wi-Fi or Airplane mode, then restart the tablet. 3. Forget the network, rejoin and re-enter the password carefully. 4. Move closer; try the other band (2.4 GHz / 5 GHz). 5. Update the tablet software. 6. Reset network settings (iPad: Settings > General > Transfer or Reset iPad > Reset > Reset Network Settings; Samsung: Settings > General management > Reset). Saved Wi-Fi and Bluetooth pairings are deleted. | EN: tablet won't connect to wifi fix ¦ iPad wifi not working DE: Tablet verbindet sich nicht mit WLAN ¦ iPad WLAN Probleme |  |
| 5 | Frozen / unresponsive | US: tablet frozen UK: tablet frozen won't respond DE: Tablet hängt / reagiert nicht | 1. Wait a minute in case an app is busy. 2. Force restart (see No. 1). 3. Charge the tablet – a very low battery can cause freezing. 4. After restart, free storage and update apps and system. 5. Android: test in Safe mode; if fine there, uninstall recent apps one by one. 6. Still freezing: back up and factory reset (No. 17). | EN: tablet frozen how to fix ¦ force restart iPad DE: Tablet eingefroren was tun ¦ Tablet reagiert nicht neu starten |  |
| 6 | Battery drains fast | US: tablet battery drains fast UK: tablet battery draining quickly DE: Tablet Akku schnell leer | 1. Check which apps use the most battery (Settings > Battery). 2. Lower brightness and enable auto-brightness. 3. Turn off Wi-Fi and Bluetooth when using the tablet offline. 4. Turn off background refresh and unneeded notifications. 5. Android: check Settings > Battery > Battery Health if available. 6. Update software and apps. 7. Batteries typically last 2 to 3 years; sudden severe drain on an older tablet means battery replacement. | EN: fix tablet battery drain ¦ Galaxy Tab battery draining fast DE: Tablet Akku schnell leer Lösung ¦ iPad Akku Laufzeit verbessern |  |
| 7 | Touchscreen not responding | US: tablet touch screen not working UK: tablet screen not responding DE: Tablet Touchscreen reagiert nicht | 1. Clean the screen with a soft, slightly damp microfibre cloth; dry hands. 2. Remove screen protector and case. 3. Unplug chargers and accessories (a faulty charger can cause phantom touches). 4. Restart or force restart. 5. Android: test in Safe mode to rule out an app. 6. Update the software. 7. Software-only fault: back up and factory reset. Cracked or dead zones need repair. | EN: tablet touch screen not responding fix DE: Tablet Touchscreen reagiert nicht |  |
| 8 | Apps keep crashing | US: tablet apps keep crashing UK: tablet apps keep closing DE: Tablet App stürzt ab | 1. Check the internet connection – unstable Wi-Fi can crash apps. 2. Update the app, then the system software. 3. Restart the tablet. 4. Clear the app cache (Android) or offload and reinstall (iPad: Settings > General > iPad Storage > app > Offload App). 5. Free up storage. 6. Android: use Safe mode to find a conflicting app. 7. One app still failing: contact its developer. | EN: apps keep crashing on tablet fix DE: Tablet App stürzt ab Lösung |  |
| 9 | Storage full | US: tablet storage full UK: tablet storage full what to delete DE: Tablet Speicher voll | 1. Check usage: iPad Settings > General > iPad Storage; Android Settings > Battery and device care > Storage. 2. Delete or offload unused apps. 3. Move photos and videos to cloud or a computer, then delete them from the tablet. 4. Clear downloads, offline media and large app caches. 5. Android with microSD slot: move media to the card. 6. Restart and confirm space was freed. | EN: free up storage on tablet ¦ iPad storage full fix DE: Tablet Speicher voll Speicher freigeben |  |
| 10 | Forgot passcode / locked out | US: forgot tablet password UK: tablet locked forgot pin DE: Tablet Passwort vergessen / entsperren | 1. For your own device only; this erases its data. 2. iPad: use Erase iPad on the lock screen (needs Apple Account password) or restore in recovery mode from a computer. 3. Samsung: Find My Mobile (Samsung account) to unlock or erase remotely. 4. Other Android: Google Find Hub / Find My Device to erase, or factory reset from recovery mode. 5. Set up again and sign in with the same Apple/Google account (activation lock / factory reset protection). 6. Restore from backup. | EN: iPad disabled how to unlock ¦ forgot tablet password reset DE: iPad deaktiviert entsperren ¦ Tablet Passwort vergessen zurücksetzen |  |
| 11 | Stuck on logo / boot loop | US: tablet stuck on logo UK: tablet stuck on startup screen DE: Tablet hängt im Startbildschirm | 1. Charge for at least 30 minutes. 2. Force restart (see No. 1). 3. Android: boot into Safe mode and remove recent apps. 4. Android: recovery mode (key combination varies) > Wipe cache partition. 5. iPad: connect to a computer and use recovery mode to update or restore. 6. Factory reset from recovery as a last resort (No. 17); if it still won't boot, contact support. | EN: tablet stuck on logo fix ¦ iPad stuck on Apple logo DE: Tablet hängt im Startbildschirm ¦ iPad Apfel Logo hängt |  |
| 12 | Bluetooth won't pair | US: tablet bluetooth won't connect UK: tablet bluetooth not pairing DE: Tablet Bluetooth verbindet nicht | 1. Turn Bluetooth off and on; restart the tablet. 2. Charge the accessory and put it in pairing mode. 3. Forget the old pairing on the tablet, then pair again. 4. Keep devices within 1 to 2 metres. 5. Update tablet software and accessory firmware. 6. Reset network settings (No. 4) before any full reset. | EN: tablet bluetooth won't pair fix DE: Tablet Bluetooth verbindet nicht |  |
| 13 | No sound | US: tablet no sound UK: tablet sound not working DE: Tablet kein Ton | 1. Raise volume; on Android check media volume separately. 2. Check mute, silent mode and Do Not Disturb. 3. Disconnect Bluetooth speakers or headphones. 4. Unplug headphones; clean the headphone jack or USB-C port. 5. Test in a different app or video. 6. Restart and update software. 7. Gently clean the speaker grille; still silent means repair. | EN: tablet no sound fix ¦ iPad no sound DE: Tablet kein Ton Lösung |  |
| 14 | Screen won't rotate | US: tablet won't rotate UK: tablet screen not rotating DE: Tablet Bildschirm dreht sich nicht | 1. Turn off rotation lock (iPad: Control Centre; Android: quick settings > Auto rotate). 2. Some apps and home screens don't rotate; test in a browser or video app. 3. Restart the tablet. 4. Update the software. 5. Samsung: tap the small rotate icon in the navigation bar if shown. | EN: tablet screen won't rotate fix DE: Tablet Bildschirm dreht sich nicht |  |
| 15 | Overheating | US: tablet overheating UK: tablet getting hot DE: Tablet wird heiß | 1. Stop using it; remove the case and move out of sunlight. 2. Unplug and let it cool naturally (no fridge or freezer). 3. Close apps and lower brightness. 4. Avoid gaming or video calls while charging. 5. Update apps and system software. 6. Overheating while idle, or a bulging back: stop using it and have the battery checked. | EN: tablet overheating fix DE: Tablet wird heiß was tun |  |
| 16 | Software update fails / stuck | US: tablet update not working UK: tablet software update stuck DE: Tablet Update funktioniert nicht | 1. Charge above 50% or keep it plugged in. 2. Check Wi-Fi is stable and free up storage (updates need spare space). 3. Restart and retry the update. 4. If stuck, force restart (No. 1) and retry. 5. iPad: update from a computer if needed. 6. Keeps failing: back up and reset (No. 17). | EN: tablet software update not working fix DE: Tablet Update funktioniert nicht |  |
| 17 | Factory reset (last resort) | US: how to factory reset tablet UK: how to reset tablet DE: Tablet zurücksetzen | 1. Back up photos, files and app data to the cloud or a computer. 2. Know your Apple or Google account password. 3. iPad: Settings > General > Transfer or Reset iPad > Erase All Content and Settings. 4. Samsung/Android: Settings > General management > Reset > Factory data reset (menu names vary). 5. Wait for the restart, set up again and restore your backup. | EN: how to factory reset iPad ¦ factory reset Samsung tablet DE: iPad zurücksetzen ¦ Samsung Tablet zurücksetzen |  |

When to seek repair

Contact the manufacturer or an authorised repairer for physical damage, a swollen battery, liquid damage, no response to charging after the steps above, or hardware faults such as a dead speaker or cracked screen.

Sources used for the issue list

Which? UK (common iPad and tablet faults); Apple Support (iPad won't turn on or is frozen); iFixit (Samsung tablet troubleshooting); Asurion (Samsung Galaxy Tablet issues); Samsung support (Galaxy Tab not charging).